# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply of Farm Equipment and Infrastructure	p 1 /…

Publication reference: ATP-SLU/TSP-025/1506/LC

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Quantity | 3. Specifications Required | 4. Specifications Offered | 5. Notes, remarks, ref to documentation | 6. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4WD Tractor (CAT 2) Engine Type: 4 – 6 cyl: Diesel Fuel capacity: (gal) 45 Hitch type: Category 2 Rear lift: (at 24”) 3000 kg Gross power: 120 PTO Power: (hp) 105 Drawbar power: 90 hp Power Take Off: Rear PTO independent Rear RPM 540 / 1000 Operator Area: ROPS with canopy Wheelbase: (in.) 100. Track width: (ft) < 6 Overall width: (ft) < 8 Wheels /Tyres: Front: 14.9*24/28 Rear: 18.4 – 34/38 |  |  |  |
| 2 | 1 | TWO - WHEEL TRACTOR Engine: 10 – 13 HP: Petrol Weight (with tiller): 340 – 460 lb Working Speeds: 3 forward, 3 reverse Working Wheel Speeds: 0.6 to 2.1 m.p.h Wheel size: 5×10 – 6.5×12 Transmission: All gear drive Axle configuration: Differential with lockout |  |  |  |
| 3 | 1 | DISC HARROW- Tandem Hitch type: Cat 1/2 Working width: (in) 72 Working depth: (in) 10 No. of disc blades: 20 Disc blade spacing: (in) 7 1/2 Disc blades type: Front: Notched; Rear: notched Disc blade size/ diameter: (in) 22 Power Requirement: 40 – 100 HP Hitch type: pulled Weight: (lbs) 2,500 |  |  |  |
| 4 | 1 | DISC BEDDER Hitch type: Cat 1/2 No. of rows: 3 Gauge wheels Number of discs: 12 Toolbar width: 12’ Bed width: 36” – 72” Power Requirement: (Hp) 45 – 100 |  |  |  |
| 5 | 1 | 3 DISC PLOUGH Hitch type: Cat 2 Number of disc blades: 3 Working width: 36” Working depth: 6” to15” Weight: 890 to 1080 lb Power requirement: (hp) 45 – 75 |  |  |  |
| 6 | 1 | TRUCK WITH CRANE MECHANISM Drive type : 6 x 4 Wheelbase: 5350 + 1300 mm Overall dimension (L x W x H): 11960 x 2500 x 3800 mm Gross weight: 25 000 Kg Curb weight: 15 000 Kg Payload weight: 10 000 Kg Engine: Rated power: 245 hp Brakes: Air brakes Gear box: 8 speed, manual Cargo box inside dimension (L x W x H): 74000 x 2300 x 800 mm Maximum lifting capacity: 10 000 kg Maximum lifting height: 12 m Boom Type: Knuckle boom Legs: 2 pieces Rotation angle: 360 degrees (All rotation) Crane weight: 3800 kg Lifting capacity at various radius: • Radius (m) o 2.5 - 10 000kg o 4.5 - 5500 o 7 - 3200 o 9 - 2300 |  |  |  |
| 7 | 1 | CONE PENETROMETER Type of assembling: Assembled with ultrasonic depth sensor and GPS Maximum small cone index: 5600kPa, 75kg Maximum Large Cone Index: 2200kPa, 75kg Resolution: 0.03kg Buad Rate/Download speed: 9600bps Screen Resolution (Characters):160x128 Battery Life: 3000mAh Small Cone Size (dia in mm, area in mm2): 160x128 Large Cone Size (dia in mm, area in mm2): 12.83, 130 Shaft Size (dia in mm): 9.53 Digital data logger and PC software Standard accessories - Battery charger - Carrying case |  |  |  |
|  | Spares And Supplies For Maintenance |  |  |  |  |
| 8 |  | 4WD Tractor (120 hp) Oil filters, fuel filters, one front tyre, one rear tyre, hydraulic oil, engine oil, seals, plugs, brakes kits SUPPLIES FOR FIRST 3 SERVICES |  |  |  |
| 9 |  | CRANE TRUCK Oil filters, fuel filters, two tyres, hydraulic oil, engine oil, seals, plugs, brake kits SUPPLIES FOR FIRST 3 SERVICES |  |  |  |
| 10 |  | TRACTOR TILLERS Filters, plugs, hydraulic oil, engine oil, seals, plugs SUPPLIES FOR FIRST 3 SERVICES |  |  |  |
| 11 | 1 | AIR COMPRESSOR: Engine: OHV Start type: Electric/with recoil backup Fuel type: Gas Maximum PSI: 175 Weight: less than 350 lbs. Cubic feet per min (CFM ) delivery @ 90 PSI: 24.4 CFM Tank type: Horizontal Pump type: Cast iron Air outlet size: ¾ inch ball low oil shut down: yes Other accessories, spares and tools for compressor •Air compressor oil - Compatible with compressor type. •Heavy duty air hose - ¾ inch by 50ft, plus other fittings for hose. •Air compressor startup kit and oil - Compatible with compressor. •Air compressor filters, lubricants - Compatible with manufacturer spec. •Air impact wrench - Compatible with CFM of the compressor. •Impact sockets - Complete set. |  |  |  |
| 12 | 1 | 5HP GENERATOR: Engine: OHV Running Watts: 6500 watts Starting watts: 8125 watts Receptacles • 20 Amp @ 120 V - 2 locking plugs • 30 Amp @120/ 240V - 1 locking plug Start type: Electric/ with recoil backup Fuel type: Gas Fuel tank capacity: 6.6 Gallons Run time at 50%: 11 hours Weight: Less than 200 lbs. Low oil shut down: yes Mobility: Mounted on heavy duty no flat wheel |  |  |  |
| 13 | 1 | TOOL KIT with: Spanners, socket set, battery charger, jumper cables, ratchets, wrench, pliers |  |  |  |